| Name of Safe Work Practice: | Sheet Metal Brake | SWP # | 00-043 |
| --- | --- | --- | --- |
| Date Developed: | Enter Date Here | Revision Date: | Enter Date Here |
| Management Signature: | Management Signature Here | Date: | Enter Date Here |
| Reviewed by JHSC/Worker Rep: | Add name of worker rep. | Date: | Enter Date Here |

Instructions:

- Review this safe work practice (SWP) annually or any time the task, equipment, or materials change.

- Do NOT perform this practice until you have been appropriately trained and authorized to do so by your supervisor.

| Check Required Personal Protective Equipment and Devices: (Delete this text and pictograms that do not apply. Add any additional personal protective equipment required). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| Eye Protection | Dust Mask | Face-Shield | Safety Footwear | Hearing Protection | Type of Gloves | Hard Hat | Protective Clothing | NIOSH Approved Respirator | Fall Protection | No Loose clothing, jewelry etc. |  |

| Required Training: (Delete this text and list all training that is required prior to completing this procedure) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type |  | Who | Topic | Description |
| Formal Training | ☐ | - All applicable personnel | SWP’s | New hire orientation/ review at least once per year thereafter |
| On the Job Training | ☒ |  |  |  |

General Safety Requirements and Instructions:

Do:

- Ensure the machine is operated or serviced only by properly trained, authorized personnel. - Wear appropriate personal protective equipment (PPE) as noted above. - Inspect tool looking for any missing, bent, or broken parts - Use your legs and arms when making bends, similar to lifting heavy objects, to avoid back strain. - Ensure the brake is set on a smooth, level floor and properly configured. - Keep all body parts and any foreign objects away from the nose bar and clamping area of the brake while in operation. - Ensure all guards and covers are in place before operating the machine. - Stay clear of the counterweight and apron swing area while operating the brake. - Keep the work area around the brake clear and clean to avoid slipping or tripping. - Adjust the set back clearance and clamping pressure for different thicknesses of material - Always bend short pieces of material in the center of the brake to equalize the stress and avoid damage. - Apply only enough clamp pressure to hold the metal in place without causing movement or slipping.

Don’t:

- Use a pipe or bar on the clamp handles for additional leverage. - Operate the machine if it has been damaged or is not operating properly. - Bend rods, nails, or wire in the Brake - Wear jewelry (watches, rings, necklaces, etc.), or loose-fitting clothing while operating or servicing the machine. - Bend hems or seams without adjusting the clamp pressure to handle the extra (double) metal thickness.