| Name of Safe Work Practice: | Handheld Angle Grinder | SWP # | 00-045 |
| --- | --- | --- | --- |
| Date Developed: | Enter Date Here | Revision Date: | Enter Date Here |
| Management Signature: | Management Signature Here | Date: | Enter Date Here |
| Reviewed by JHSC/Worker Rep: | Add name of worker rep. | Date: | Enter Date Here |

Instructions:

- Review this safe work practice (SWP) annually or any time the task, equipment, or materials change.

- Do NOT perform this practice until you have been appropriately trained and authorized to do so by your supervisor.

| Check Required Personal Protective Equipment and Devices: (Delete this text and pictograms that do not apply. Add any additional personal protective equipment required). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| Eye Protection | Dust Mask | Face-Shield | Safety Footwear | Hearing Protection | Type of Gloves | Hard Hat | Protective Clothing | NIOSH Approved Respirator | Fall Protection | No Loose clothing, jewelry etc. |  |

| Required Training: (Delete this text and list all training that is required prior to completing this procedure) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type |  | Who | Topic | Description |
| Formal Training | ☐ | - All applicable personnel | SWP’s | New hire orientation/ review at least once per year thereafter |
| On the Job Training | ☒ |  |  |  |

General Safety Requirements and Instructions:

Do:

- Perform a pre-use inspection, checking for any missing, bent, or damaged parts, including the cord, handles, and guards. - Ensure the plug is equipped with a ground fault interrupter when operating in damp conditions. - Confirm that accessories are rated for the speed recommended on the tool’s warning label. - Carefully check the wheel for cracks or damage. Replace any cracked or damaged wheels immediately. - Follow applicable “Electrical Lockout” procedures when changing accessories. - Before using the tool, let it run and observe for vibration or wobbling, which could indicate poor installation or an unbalanced wheel. - Hold the tool so that sparks fly away from you, others, and any flammable materials. - Use clamps or another secure method to support the workpiece on a stable platform. - Ensure the wheel is not contacting the workpiece before turning the switch on. - Keep hands away from rotating parts. - Ensure the switch is in the off position before connecting to the power source, battery pack, or while picking up or carrying the tool.

Don’t:

- Use tool if switch does not turn it on or off. - Operate power tools in explosive atmospheres - Wear loose clothing or leave long hair unconfined around rotating parts. - Use the cord to carry the tool or pull the plug from an outlet. - Overreach; always maintain proper footing and balance. - Leave the tool running unattended. Operate the tool only when hand-held. - Actuate the shaft lock when the spindle is moving. - Bang or hit grinding disc or wheel onto work. - Use cut-off wheel for side grinding - Use a damaged or unbalanced brush (wire cup). - Touch the workpiece immediately after operation; it may be extremely hot and could burn your skin.