| Name of Safe Work Practice: | Skid Steer | SWP # | 00-049 |
| --- | --- | --- | --- |
| Date Developed: | April 13, 2026 | Revision Date: | Enter Date Here |
| Management Signature: | Management Signature Here | Date: | Enter Date Here |
| Reviewed by JHSC/Worker Rep: | Add name of worker rep. | Date: | Enter Date Here |

Instructions:

- Review this safe work practice (SWP) annually or any time the task, equipment, or materials change.

- Do NOT perform this practice until you have been appropriately trained and authorized to do so by your supervisor.

| Check Required Personal Protective Equipment and Devices: (Delete this text and pictograms that do not apply. Add any additional personal protective equipment required). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| Eye Protection | Dust Mask | Face-Shield | Safety Footwear | Hearing Protection | Type of Gloves | Hard Hat | Protective Clothing | NIOSH Approved Respirator | Fall Protection | No Loose clothing, jewelry etc. |  |

| Required Training: Valid 3-year certificate (8 hours theory and hands-on) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type |  | Who | Topic | Description |
| Formal Training | ☒ | Workers operating skid steers | SWP’s should follow manufacturer’s instructions | Operators must recertify every three years. |
| On the Job Training | ☒ |  |  |  |

General Safety Requirements and Instructions:

Do:

- Only trained, competent, and authorized operators may operate the machine - Operators must be familiar with the manufacturer’s operator manual - Wear required PPE at all times - Perform and document a pre-use inspection - Ensure all safety features are functional (e.g., lights, seatbelts, mirrors, horn) - Ensure attachments are properly installed, secured, and locked - Check gauges and indicators to confirm the machine is within normal operating limits - Test all controls (steering, forward/reverse, lift arms, attachment functions) before use - Inspect ROPS/FOPS to confirm they are in place and not modified or damaged - Report and tag out any defective or unsafe equipment; notify supervisor - Maintain three-point contact when entering or exiting - Adjust the seat so controls are easy to reach - Ensure the area around the machine is clear of obstructions and personnel - Start and operate the machine only from the operator’s seat with controls in neutral and parking brake engaged - Seatbelt must be worn at all times while operating - Keep hands and feet on controls - Operate at safe speeds, adjusting for site conditions, environmental factors, and hazards (e.g., slopes, ice, mud, rain, congestion) - Travel straight up and down slopes; keep loads low and stable - Maintain awareness of underground and overhead utilities and maintain required clearances - Keep a clear travel path, follow the Traffic Management Plan (TMP), maintain safe distances, and use a spotter with clear communication in congested areas - Be familiar with the site emergency response plan and alternate exit options (e.g., rear window, with breaker/cutter available) - When shutting down or leaving seat - Ensure machine is on even ground - Lower attachments to the ground - Place controls in neutral and set parking brake - Shut off engine - Follow lockout/tagout (LOTO) procedures when servicing, cleaning, or clearing jams - Ensure the machine is de-energized before performing any work or refueling

Don’t:

- Do not operate equipment without proper training or authorization - Do not operate under the influence of drugs, alcohol, or impairment - Never allow passengers on the machine - Never use buckets, forks, or attachments to lift or carry people - Do not use raised equipment as a work platform - Do not operate equipment that is damaged, malfunctioning or tagged out - Do not disable safety devices, ROPS, or seat belts - Do not exceed safe speeds or operate recklessly - Avoid turning on slopes or operating across steep inclines - Do not carry loads that obstruct visibility - Do not operate near workers without ensuring clear separation and a communication plan - Do not back up without checking surroundings or using a spotter - Do not attempt to adjust, clean, lubricate, or clear obstructions while the machine is running or moving - Do not leave the machine unattended with the engine running or attachments raised