| Name of Safe Work Practice: | Powered Nail Guns (Pneumatic, Gas-Actuated, and Electric) | SWP # | 00-050 |
| --- | --- | --- | --- |
| Date Developed: | Enter Date Here | Revision Date: | Enter Date Here |
| Management Signature: | Management Signature Here | Date: | Enter Date Here |
| Reviewed by JHSC/Worker Rep: | Add name of worker rep. | Date: | Enter Date Here |

Instructions:

- Review this safe work practice (SWP) annually or any time the task, equipment, or materials change.

- Do NOT perform this practice until you have been appropriately trained and authorized to do so by your supervisor.

| Check Required Personal Protective Equipment and Devices: (Delete this text and pictograms that do not apply. Add any additional personal protective equipment required). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| Eye Protection | Dust Mask | Face-Shield | Safety Footwear | Hearing Protection | Type of Gloves | Hard Hat | Protective Clothing | NIOSH Approved Respirator | Fall Protection | No Loose clothing, jewelry etc. |  |

| Required Training: (Delete this text and list all training that is required prior to completing this procedure) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type |  | Who | Topic | Description |
| Formal Training | ☐ | - All applicable personnel | SWP’s should follow manufacturer’s instructions, manual and warning labels | New hire orientation/ review at least once per year thereafter |
| On the Job Training | ☒ |  |  |  |

General Safety Requirements and Instructions:

Do:

- Read the manual — be familiar with the machine you're working with. - Perform a pre-use inspection, checking the nail gun before each use, confirming the nose contact element (safety tip), trigger guard, and depth adjustment are intact and functional. - Establish exclusion zones where feasible. - Ensure that if working at heights tool is secured with a lanyard and other working at heights controls are in place. - Ensure you have been trained on the specific make, model, and trigger type of the nail gun before operating it. - Ensure proper ergonomic positioning when operating (MSIs - awkward positions/repetitive strain) - PNEUMATIC: Set air compressor to manufacturer’s recommended pressure; do not exceed the tool’s maximum rated pressure. Drain moisture from the tank daily. Lubricate with manufacturer-approved oil unless the tool is oil-free. - GAS-ACTUATED: Check the fuel cell for leaks or expiry and verify battery charge. Operate in a well-ventilated area only – combustion gases are produced on each firing cycle. - ELECTRIC: Confirm the battery is fully charged and properly latched, or that the power cord is undamaged and grounded. Keep the cord behind you during operation. - Use only the correct nail type, gauge, and length specified by the manufacturer for the tool and the task. - Place the nose of the nail gun firmly against the work surface before pulling the trigger every time. - Keep hands and fingers well clear of the nail path. Use clamps to secure workpieces when your hands would otherwise be near the nailing zone. - Be aware of what is on the other side of the material being fastened. Nails can penetrate thin boards and injure people or strike utilities behind the surface. - Disconnect from the power source (air hose disconnected at tool/fuel cell and battery removed / power cord unplugged) before clearing jams, reloading, or performing any maintenance. - Remove fasteners from the magazine, engage any trigger safety lock, and store in the manufacturer’s case or a secured tool area at the end of the shift. Report any defects to your supervisor.

Don’t:

- Point a nail gun at yourself or anyone else, even when disconnected from its power source. - Carry a nail gun with your finger on the trigger. - Defeat, tie back, or bypass the nose contact safety element. - Do not overreach while operating nail gun at heights. - Use the wrong nail type, gauge, or length for the tool. - Clear a jam, reload, or perform maintenance while the tool is connected to its power source. - Allow untrained workers to handle or operate a nail gun. - Use a gas-actuated nail gun in confined spaces or near flammable vapours or ignition sources. - Bounce-fire (hold trigger while repeatedly bumping the nose contact against a surface) unless specifically trained and the task requires contact-trip operation. - Modify, repair, or override any safety feature of the nail gun. - PNEUMATIC: Never use compressed air to blow debris off clothing, skin, or other people. - Do not operate gas actuated tools in poorly ventilated spaces